Device Drivers for DOS/Windows

[Introduction:]

In SDMS 3.0, the SCSI BIOS for each of the NCR family of PC SCSI chips is

capable of mapping SCSI hard disk drives behind any non-SCSI hard disk drives

(IDE, ESDI, etc.) within the same system. A driverless solution will allow up

to eight hard drives (SCSI and non-SCSI) to be connected under DOS 5.0 and

above. The SCSI BIOS also supports removable drives with 512-byte sectors as

long as the media is in the drive at boot-up and remains in the drive during

system operation. Low level Virtual DMA Services (VDS) are supported by the

SCSI BIOS. Therefore, to gain maximum performance, any double buffer option

provided by disk caching software (such as Microsofts SMARTDRV.EXE) should

be disabled for all drives handled through SDMS. For full VDS support,

including features such as scatter-gather, DOSCAM must be loaded in the

CONFIG.SYS file. Connecting peripherals other than hard disk drives will

also require loading the appropriate driver(s). Some of the drivers work

together, and some are capable of direct communication with an NCR SCSI BIOS

The following sections list these drivers, their features, and their loading

requirements.

[DOSCAM.SYS]

Features

Full CAM 3.0 support

Synchronous negotiation (including fast SCSI)

Full Virtual DMA Services (VDS) support (including scatter-gather)

Tagged command queuing

Supports multiple host adapters

Supports multi-threading

Disconnect/Reselect

Description

DOSCAM is the full-featured CAM 3.0 DOS driver. It has all of the features

and functionality  of MINICAM, but whenever  one  of the above features is

required  DOSCAM should  be loaded instead. Synchronous  negotiation  will

allow data transfers of  5 MB/s (up to 10 MB/s  for fast SCSI, and 20 MB/s

for fast wide SCSI). Tagged command queuing provides a performance improv-

ement under multi-threaded I/O operating systems.

Installation

1.Use the COPY command to copy the appropriate driver from the SDMS SCSI

Drivers disk to your boot disk.

2.Add this line to your systems CONFIG.SYS file:

DEVICE=C:<PATH>DOSCAM.SYS

Note:If the computer system is using the DOS HIMEM.SYS and EMM386 memory

managers, the device driver should be listed after those entries.

Command Line Options

The DOSCAM device driver has several embedded functions which can be

accessed via switches on the command line.

1.Using the /RAMCOPY Option

RAMCOPY is an embedded command which tells the drivers to load the ROM

on the controller card into RAM, thereby increasing the performance of

the SCSI ROM.

ex.DEVICE=C:<PATH>DOSCAM.SYS /RAMCOPY

Upon boot, the RAMCOPY command will be executed & ROM will load into RAM.

The first ROM will be copied into expanded memory if available.

2.Using the /T=n Option

DOSCAM uses a time-out mechanism to detect certain errors. When DOSCAM

issues a command to a SCSI device, a timer is started. If the timer

expires before the command completes, DOSCAM assumes that something has

gone wrong with the device, and takes steps to recover. The default

value for this time-out is no time-out. The time-out value can be

specified as an argument to DOSCAM in the CONFIG.SYS file. This argument

consists of a /T=n parameter, with n being the number of seconds you

want it to wait before timing out. For example, the line in CONFIG.SYS

that loads DOSCAM would look like this:

DEVICE=C:<PATH>DOSCAM.SYS /T=4

with  /T being the time-out option and 4 being the number of seconds

you want it to wait before timing out.

3.Using the /ASK Option

This option prompts the user at system boot-up whether to load DOSCAM

or not. For example, the line in CONFIG.SYS that loads DOSCAM would

look like this:

DEVICE=C:<PATH>DOSCAM.SYS /ASK

Troubleshooting

1.THE COMPUTER LOCKS UP AND WILL NOT COMPLETE BOOTING FROM A NON-SCSI HARD

DISK DRIVE. Refer to the drive manufacturer's user manual.

2.THE COMPUTER LOCKS UP AND WILL NOT COMPLETE BOOTING FROM A SCSI HARD

DISK DRIVE.

a.Is the SCSI BIOS seen during boot?

YES, Go to b.

NO,  Power down all units in the system.

Remove all SCSI cables.

Boot system.

Is the SCSI BIOS seen during boot?

YES, Power down all units in the system.

Reconnect and check all cable and power connections.

Boot system.

Go to a.

NO,  Power down all units in the system.

Reseat the host bus adapter.

Check jumper settings.

For example, settings for:IRQ,SCSI chip address

Check BIOS configuration.

Boot system.

Go to a.

b.Does the SCSI BIOS see the bootable SCSI drive?

YES, Go to c.

NO,  Power down all units in the system.

Make sure the hard drives have different ID numbers

(boot drive should have lowest ID).

Make sure both ends of the SCSI bus are terminated.

Check all cable and power connections.

Check BIOS configuration.

Boot system.

Go to a.

c.If boot is still unsuccessful, go to the following item.

3.THE DEVICE DRIVER DOES NOT RECOGNIZE ONE OF THE NON-BOOT SCSI PERIPHERALS (system may lock up).

a.Make sure the drivers were installed in the correct sequence.

b.Make sure the drivers CONFIG.SYS line has the correct path to the

drivers.

c.Power down all units in the system.

d.Make sure the hard drives have different ID numbers (boot drive should

have lowest ID).

e.Make sure both ends of the SCSI bus are terminated.

f.Check all cable and power connections.

g.Boot the system.

[SCSIDISK.SYS]

Features

Can communicate directly with the SCSI BIOS

Support for removable media devices

Supports non-512-byte sectors (1024, 2048, 4096)

Multiple logical unit number (LUN) support

Supports multiple host adapters in a single system

Write protection

Description

SCSIDISK.SYS is needed when connecting more than seven drives under DOS 5.0

and above. It is also required for drives with-512-byte sectors and for

removable drives if the user wants to change the media.

SCSIDISK can talk directly to the SCSI BIOS, or will work through  DOSCAM.

Installation

1.Use the COPY command to copy the appropriate driver from the SDMS SCSI

Drivers disk to your boot disk.

2.Add this line to your systems CONFIG.SYS file:

DEVICE=C:<PATH> SCSIDISK.SYS

Note:If the computer system is using the DOS HIMEM.SYS and EMM386 memory

managers, the device driver should be listed after those entries.

If the DOSCAM driver is also desired, list the drivers in this sequence:

DEVICE=C:<PATH> DOSCAM.SYS

DEVICE=C:<PATH> SCSIDISK.SYS

Command Line Options

The SCSIDISK device driver has several embedded functions which can be

accessed via switches on the command line. These options are described

below.

1.Using the /ASK Option

This option prompts the user at system boot-up whether to load SCSIDISK

or not. For example, the line in CONFIG.SYS that SCSIDISK would look like

this:

DEVICE=C:<PATH>SCSIDISK.SYS /ASK

2.Using the /UNITS=n Option

SCSIDISK allows the use of removable media, such as cartridge hard drives.

The /UNITS=n option reserves the needed part by setting the number of

logical drive units (not LUNs) reported to MS-DOS to n. The maximum value

for n is 8. This is useful when the system is started without any media

in the drive, or when media which has more than one volume partition is

insert after boot-up. The /UNITS option also allows continued use of the

removable media drive (without rebooting) if the media is exchanged for

a different media with a different number of volumes on it. A system with

the SCSIDISK device driver installed allows booting from a removable

media volume; however, the removable media cartridge  cannot be removed

until after system shutdown. If removable of the media cartridge is

desired without shutting down the system, the system must be booted

without the removable media cartridge inserted in the drive. To use this

option, the line in CONFIG.SYS that loads SCSIDISK should look like this:

DEVICE=C:<PATH>SCSIDISK.SYS /UNITS=n

3.Using the /PROTECT=x Option

This option write-protects MS-DOS volumes on a SCSI hard drive. This

option does not work with an IDE drive. Any write at will return a Write

Protected Media error code. To enable the write protection option, the

line in CONFIG.SYS that loads SCSIDISK should look like this:

DEVICE=C:<PATH>SCSIDISK.SYS /PROTECT=x

where x is the DOS volume, such as C, D, E, F, G, ..., Z. The x represents

the volume letter(s) without any colons. For example:

DEVICE=C:<PATH>SCSIDISK.SYS /PROTECT=D

This protection is not guaranteed as it is possible to bypass the

protection with low-level programs, such as FDISK whichs INT 13. Another

possible bypass is to boot from a floppy disk without the write protection

option enabled.

Troubleshooting

1.THE COMPUTER LOCKS UP AND WILL NOT COMPLETE BOOTING FROM A NON-SCSI HARD

DISK DRIVE. Refer to the drive manufacturers user manual.

2.THE COMPUTER LOCKS UP AND WILL NOT COMPLETE BOOTING FROM A SCSI HARD DISK

DRIVE.

a.Is the SCSI BIOS seen during boot?

YES, Go to b.

NO,  Power down all units in the system.

Remove all SCSI cables.

Boot system.

Is the SCSI BIOS seen during boot?

YES, Power down all units in the system.?

Reconnect and check all cable and power connections.

Boot system.

Go to a.

NO,  Power down all units in the system.

Reseat the host bus adapter.

Check BIOS configuration.

Boot system.

Go to a.

b.Does the SCSI BIOS see the bootable SCSI drive?

YES, Go to c.

NO,  Power down all units in the system.

Make sure the hard drives have different ID numbers (boot drive

should have lowest ID).

Make sure both ends of the SCSI bus are terminated.

Check all cable and power connections.

Check BIOS configuration.

Boot system.

Go to a.

c.If boot is still unsuccessful, go to the following item.

3.THE DEVICE DRIVER DOES NOT RECOGNIZE ONE OF THE NON-BOOT SCSI PERIPHERALS

(system may lock up).

a.Make sure the drivers were installed in the correct sequence.

b.Make sure the drivers CONFIG.SYS line has the correct path to the drivers.

c.Power down all units in the system.

d.Make sure the hard drives have different ID numbers (boot drive should have lowest ID).

e.Make sure both ends of the SCSI bus are terminated.

f.Check all cable and power connections.

g.Boot the system.

[CDROM.SYS]

Features

Can communicate directly with the SCSI BIOS

Compatible with Microsofts CD-ROM Extension 2.21 and above

Description

CD-ROM.SYS is needed whenever a CD-ROM device is connected on the SCSI bus.

It must be loaded in conjunction with Microsoft CD-ROM Extension 2.21

(MSCDEX.EXE). At the beginning of this chapter, CDROM is capable of

communicating directly with the SCSI BIOS, or will work through DOSCAM.

Installation

1.Use the COPY command to copy the appropriate driver from the SDMS SCSI

Drivers disk to your boot disk.

2.Add CDROM to your CONFIG.SYS file. It goes in after MINICAM or DOSCAM and

SCSIDISK (if these drivers are also being used), in this order:

DEVICE=C:<PATH>DOSCAM.SYS

DEVICE=C:<PATH>SCSIDISK.SYS

DEVICE=C:<PATH>CDROM.SYS /D:NAME

Note:The /D: is not a drive letter designation; it indicates the name you

wish assigned to your CD-ROM. The NAME must be included, and can be any

combination of up to 8 characters.

3.To insure that sufficient drive letters are available to identify all

devices connected to the SCSI chain, add the MS-DOS LASTDRIVE command to

the CONFIG.SYS file:

LASTDRIVE=x

with x specifying a drive letter in the range C through Z. The letter

assigned to LASTDRIVE represents the last valid drive MS-DOS is able to

recognize and also represents the maximum number drives available. For

example, LASTDRIVE=K allows access to eleven (11) logical drives. For

further details about LASTDRIVE, consult your MS-DOS manual.

4.Unless your CD-ROM access software specifies otherwise, Microsofts CD-ROM

Extension (MSCDEX.EXE) should execute from the AUTOEXEC.BAT file in order

to access your drive. Add the following line to our AUTOEXEC.BAT file:

MSCDEX /D:NAME

For example, if:

DEVICE=C:<PATH>CDROM.SYS /D:MY_CD

is in CONFIG.SYS, then:

MSCDEX /D:MY_CD

should be in the AUTOEXEC.BAT file.

5.Check installation instructions for the CD-ROM drive itself. There may be

other parameters necessary to include with MSCDEX.

6.When MSCDEX is loaded during the AUTOEXEC.BAT file execution, a message

is returned assigning a drive letter to the CD-ROM drive. For example:

DRIVE E = DRIVER MY_CD UNIT 0

This informs you that the CD-ROM drive is recognized and ready for use.

Command Line Options

The CDROM device driver has one embedded function available, which can be

accessed via a switch on the command line.

1.Using the /ASK Option

This option prompts the user at initialization time whether to load

CD-ROM or not. For example, the line in CONFIG.SYS adds SCSIDISK would

look like this:

DEVICE=C:<PATH>CDROM.SYS /ASK

Troubleshooting

THE CD-ROM DRIVE IS NOT SEEN AT BOOT TIME, OR THE SYSTEM LOCKS UP.

a.Make sure the driver is installed and in the correct sequence.

b.Make sure the drivers CONFIG.SYS line has the correct path to the driver.

c.Make sure MSCDEX, in the AUTOEXEC.BAT, has the same drive name as the CDROM driver in the CONFIG.SYS file.

d.Make sure there is no ID or drive letter designation conflict.

e.Power down all units in the system.

f.Check the cable and power connections.

g.Make sure both ends the SCSI chain are terminated.

h.Check the compatibility of the media with the operating system.

[ASPICAM.SYS]

Features

Supports Advanced SCSI Programming Interface (ASPI) applications

Description

ASPICAM is an ASPI manager which provides compatibility between CAM and

popular ASPI applications. At the beginning of this chapter, ASPICAM

communicates with the SCSI BIOS only through DOSCAM. It must be loaded (after

DOSCAM) whenever running an ASPI application. Some of the ASPI applications

supported under DOS include CorelSCSI!, Sytos Plus, and Central Point Tape

Backup. These provide support for tape, WORM, scanner, and other SCSI

peripherals.

Installation

1.Use the COPY command to copy the ASPICAM.SYS driver from the SDMS SCSI

Drivers disk to your boot disk.

2.Add ASPICAM to your systems CONFIG.SYS file after DOSCAM. For example,

the lines in your CONFIG.SYS might look like this:

DEVICE=C:<PATH>DOSCAM.SYS

DEVICE=C:<PATH>ASPICAM.SYS

Command Line Options

There are no command line options with ASPICAM.SYS.

Troubleshooting

1.SYSTEM LOCKS UP AT BOOT TIME.

a.Make sure no other ASPI manager is loaded.

b.Check for correct loading sequence in the CONFIG.SYS file.

c.Boot the system.

2.THE DEVICE DRIVER DOES NOT RECOGNIZE ONE OF THE NON-BOOT SCSI PERIPHERALS (system may lock up).

a.Make sure the drivers were installed in the correct sequence.

b.Make sure the drivers CONFIG.SYS line has the correct path to the

drivers.

c.Power down all units in the system.

d.Make sure the hard drives have different ID numbers.

e.Make sure both ends of the SCSI chain are terminated.

f.Check all cable and power connections.

g.Boot the system.

[Important Additional Information]

SCSI Hard Disk Drive Installation

1.Turn off the power to your computer system

2.Install the hard drive(s) being added to your system following the

instructions provided by the device vendors.

3.Make sure the device drivers are installed on the bootable drive,

either the A: drive (floppy disk) or the C: drive (hard disk), and

properly listed in the CONFIG.SYS file. Then switch on power to the

system.

4.After the computer boots, run the FDISK utility to partition the hard

drive(s) being installed. If the drive will be used to boot, be sure to

set the partition active. For details about the FDISK program, consult

your MS-DOS manual.

5.When FDISK completes partitioning and your computer has rebooted, run the

FORMAT program. For details about the program, consult your MS-DOS manual.

Be sure to use the /S option to copy the operating system files if you

want the disk being formatted to operate as the boot drive.

6.If the new hard disk drive will be used to boot, put the CONFIG.SYS file

that calls the device drivers and the device driver files on the disk.

7.To insure that sufficient drive letters are available to identify all

devices connected to the SCSI chain, add the MS-DOS LASTDRIVE command

to the CONFIG.SYS file:

LASTDRIVE=x

with x specifying a drive letter in the range C through Z. The letter

assigned to LASTDRIVE represents the last valid drive. MS-DOS is able to

recognize and also represents the maximum number drives available under

MS-DOS. For example, LASTDRIVE=K allows access to eleven (11) logical

drives. For further details about LASTDRIVE, consult your MS-DOS manual.

The SCSI hard drive(s) added to your system can now be used.

Troubleshooting

1.THE COMPUTER LOCKS UP AND WILL NOT COMPLETE BOOTING.

a.Is the SCSI BIOS seen during boot?

YES, Go to b.

NO,  Power down all units in the system.

Remove all SCSI cables.

Boot system.

Is the SCSI BIOS seen during boot?

YES, Power down all units in the system.

Reconnect and check all cable and power connections.

Boot system.

Go to a.

NO, Power down all units in the system.

Reseat the host bus adapter.

Check jumper settings.

For example, settings for: IRQ, SCSI chip address

Check BIOS configuration.

Boot system.

Go to a.

b.Does the SCSI BIOS see the bootable SCSI drive?

YES, Go to c.

NO,  Power down all units in the system.

Make sure the hard drives have different ID numbers (boot disk

should have lowest ID).

Make sure both ends of the SCSI bus are terminated.

Check all cable and power connections.

Check BIOS configuration.

Boot system.

Go to a.

c.If boot is still unsuccessful, go to the following item.

2.MS-DOS WILL NOT BOOT FROM THE HARD DISK.

a.Make sure the primary MS-DOS partition is set active (with FDISK).

b.Make sure the /S parameter was used when formatting the boot partition.

c.Make sure the boot drive has the lowest SCSI ID number on the SCSI

chain.

3.THE DEVICE DRIVER DOES NOT RECOGNIZE ONE OF THE NON-BOOT SCSI PERIPHERALS (system may lock up).

a.Make sure the drivers were installed in the correct sequence.

b.Make sure the drivers's CONFIG.SYS line has the correct path to the

drivers.

c.Power down all units in the system.

d.Make sure the hard drives have different ID numbers (boot disk should

have lowest ID).

e.Make sure both ends of the SCSI bus are terminated.

f.Check all cable and power connections.

g.Boot the system.

Assignment of Drive Letters

The MS-DOS operating system assigns drive letters to primary partitions

first. After the primary partitions have been assigned drive letters the

logical partitions are assigned drive letters. To assume that the drive

letter designations will follow consecutively from device to device within

a PC system. An Example:

A PC system is configured with an IDE hard disk as the boot drive, a SCSI

hard disk, and a CD-ROM drive. The IDE drive have free partitions: one

primary and two logical. The SCSI hard disk has two partitions: one primary

and one logical. The SCSI hard disk is assign ID one, and the CD-ROM is

ID four. The distribution of the drive letters will be:

A:3 1/2" floppy drive

B:5 1/4" floppy drive

C:IDE primary partition

D:SCSI primary partition

E:IDE first logical partition

F:IDE second logical partition

G:SCSI logical partition

H:CD-ROM

[EMM386.EXE]

EMM386.EXE, the expanded memory manager included with MS-DOS 5.0, 6.0 and

Windows 3.1, is compatible with the NCR SCSI BIOS and device driver

software. EMM386.EXE uses extended memory to simulate expanded memory for

those applications that use it. EMM386.EXE also allows programs and device

drivers to load into the upper memory area. The upper memory area is that

384K immediately following the 640K of conventional memory, normally

reserved for running the computers hardware. With EMM386.EXE the device

drivers can be loaded into high memory.

Using EMM386.EXE

In your CONFIG.SYS file, the line:

DEVICE=C:<PATH>EMM386.EXE

must come before the device drivers. The ordering is as follows:

DEVICE=C:<PATH>HIMEM.SYS

DOS=HIGH, UMB

DEVICE=C:<PATH>EMM386.EXE NOEMS

DEVICEHIGH=C:<PATH>DOSCAM.SYS

DEVICEHIGH=C:<PATH>SCSIDISK.SYS

HIMEM.SYS must be included in the CONFIG.SYS file before the device command

for EMM386.EXE.Several options are also available for use with EMM386.EXE.

Two of the most useful are RAM and NOEMS. The RAM option provides access to

both expanded memory and the upper memory. The NOEMS option provides access

to the upper memory area but not to expanded memory. Refer to the DOS and

WINDOWS manuals for more information on memory management options.

Troubleshooting

1.WHILE USING THE XCOPY /V COMMAND TO COPY FILES, YOU GET AN ERROR READING

DRIVE C:.

a.Put d=64 after DEVICE=C:<PATH>EMM386.EXE in your CONFIG.SYS file. The

d=nnn command specifies how much memory size be reserved for buffered

direct memory access (DMA). Using d=64 indicate that 64K is the largest

DMA transfer that will occur while EMM386.EXE is active.

[Installing QEMM on a SCSI Drive]

As more and more application programs use greater amounts of RAM, the ability

to load device drivers into available high memory has become more important.

Quarterdecks Expanded Memory Manager 386 (QEMM) was developed for this

purpose and NCR has developed its device drivers to be compatible with QEMM.

During the installation of QEMM, the installation program will run the

OPTIMIZE feature. OPTIMIZE will recognize that the NCR drivers DOSCAM and/or

SCSIDISK are device drivers and modify the CONFIG.SYS file to load these

drivers into high memory. If problems are encountered during the OPTIMIZE

phase, reboot from a floppy boot disk and edit the CONFIG.SYS file to ensure

that DOSCAM and/or SCSIDISK are loaded into low memory. If problems are still

encountered during the OPTIMIZE phase, modify the CONFIG.SYS file so that

DOSCAM and/or SCSIDISK are loaded first.

[WINDOWS 3.0 & 3.1]

NCR SCSI drivers are compatible with Windows 3.0 and 3.1. No additional

drivers or configuring is needed.