This disk contains the current SDMS 3.0 drivers, separated into

sub-directories by operating system.  The following sub-directories

exist on this disk:

\DOS      This directory contains the SDMS 3.0 DOS/WINDOWS drivers.

\NET3X    This directory contains the SDMS 3.0 NetWare 3.X drivers.

NCRSDMS.* The files contains the WINDOWS NT Miniport Driver.

Please note that the contents of this directory must be

copied to the root directory of whatever disk is used to

install the WINDOWS NT Miniport driver.

OS2CAM.*  The files contains the SDMS 3.0 OS/2 drivers.

The following product versions are included on this disk:

"SCSI-3.01.00"

"DOSCAM-3.01.00"

"CDROM-3.01.01"

"ASPICAM-3.01.00"

"ASPICAM-3.01.01"

"MINICAM-3.01.00"

"SCSIXPT-3.01.00"

"SCSIDSK-3.01.00"

"WINNT-3.01.00"

"OS2CAM-3.00.09"

SCSI INSTALLATION GUIDE FOR MS-DOS

(Including QEMM and Windows 3.0 & 3.1)

Release 3.00

(This document is published by Microsoft Word 3.X.)

December 1992

Information in this document is subject to change without notice and does not represent a commitment on the part of Ballard Synergy Corporation. The software described in this document is furnished under a license agreement or nondisclosure agreement. It

Copyright Ballard Synergy Corporation, 1992. All rights reserved. Simultaneously published in the United States and Canada.

CAMcore, CAMpliant, and CAMAssist are registered trademarks of Ballard Synergy Corporation.

SCO UNIX is a registered trademark of The Santa Cruz Operation.

Microsoft, MS-DOS, MS-DOS 3.3, MS-DOS 4.01, MS-DOS 5.0, Windows 3.0, Windows 3.1, and EMM386 are registered trademarks of Microsoft Corporation.

IBM, Presentation Manager, Operating System/2, and OS/2 are registered trademarks of International Business Machines Corporation.

Netware is a registered trademark of Novell.

QEMM is a registered trademark of Quarterdeck Office Systems.

SYTOS and SYTOS PLUS are registered trademarks of Sytron.

Advanced SCSI Programming Interface and ASPI are trademarks of Adaptec, Inc.

The SCSI controller performs certain tasks automatically (controlling hard disk drives, CD-ROM drives, tape drives, removable media), relieving your computer from the burden of doing so. This allows your computer to spend more time helping you with your t

Hardware connected to the computer that communicates with it are called peripheral devices. The SCSI host bus adapter can be connected to up to seven SCSI peripheral devices, such as SCSI disk drives, SCSI CD-ROM players, SCSI scanners, and SCSI tape driv

This SCSI host bus adapter will also function as a secondary adapter in a system which already has a promary hard drive controller card (IDE, ESDI, ST506). Only SCSI host bus adapters will do this, opening up a whole new world of system configuration.

This SCSI host bus adapter installation package is for systems using MS-DOS (including QEMM and Windows 3.0 & 3.1). System driver packages for Netware 2.X, Netware 3.X, OS/2, and SCO Unix are also available.

Basic Rules for SCSI Host Bus Adapter and Device installation

1. BOTH THE SCSI HOST BUS ADAPTER, AND THE LAST DEVICE ON THE SCSI CHAIN MUST BE TERMINATED.

Refer to the hardware manuals for the devices and the host bus adapter to determine where and what the terminators are.

2. EACH SCSI DEVICE MUST BE CONFIGURED WITH A DIFFERENT ID NUMBER.

Refer to the hardware manuals for the devices and the host bus adapter to locate where the jumpers or dip switches are for setting ID numbers. Usually the host bus adapter is ID 7. The devices are then set at IDs 0 through 6. The bootable hard drive shoul

3. THE RED OR BLUE LINE ON A STANDARD SCSI CABLE (OR THE BLACK LINE ON ONE END OF A MULTI-COLORED (RAINBOW) SCSI CABLE)

DESIGNATES PIN ONE ON THE CABLE CONNECTOR AND MUST CONNECT TO PIN ONE ON THE DEVICE OR HOST BUS ADAPTER CONNECTOR.

Refer to the hardware manuals for the devices and the controller to locate pin one of the connector.

4. IF THE SYSTEM ALREADY HAS AN INTERNAL BOOTABLE HARD DRIVE (IDE, ESDI, ST506), THE DRIVERS FOR THE SCSI DEVICE MUST RESIDE ON THE INTERNAL BOOTABLE HARD DRIVE.

Please refer to the section entitled INSTALLING THE DRIVE(S) of this manual.

5. SCSI HARD DRIVES ARE ALREADY LOW-LEVEL FORMATTED. THE INSTALLER NEEDS TO FDISK AND FORMAT THE DRIVE.

6. WHEN USING THE SCSI HOST BUS ADAPTER AS THE SECONDARY ADAPTER CARD IN A SYSTEM WITH A PRIMARY CONTROLLER CARD, YOU MUST ASSIGN EACH CARD A SEPARATE BASE ADDRESS.

When installing multiple hard drive adapters in one system, each card must be set to a different base address. C800 for the primary card, CC00 for the second card, etc. You can use different base address if you like, as long as the primary adapter has a l

===============================================================================

NCR SCSI DRIVERs FOR PCI-M/B

Release data: Sep. 07,1993

Reason for update: Match the Intel PCi BIOS specification Rev. 2.0G & enhance

the system performance

file name          notice                              version

-------------------------------------------------------------------------------

SCSI_DSK.DSK    ;READY For NOVELL SERVER.            (3.01.00)

SCSI_SIM.NLM    ;READY For NOVELL SERVER.            (3.01.00)

SCSI_XPT.NLM    ;READY For NOVELL SERVER.            (3.01.00)

README.DOC      ;READY For help and instruction.

DOSCAM.SYS      ;READY For DOS and Windows.          (3.01.00)

SCSIDISK.SYS    ;READY For DOS and Windows.          (3.01.00)

CDROM.SYS       ;READY For CDROM.                    (3.01.00)

ASPICAM.SYS     ;READY For SYTOS tape backup.        (3.01.00)

ASPICAM.NLM     ;READY For NOVELL SERVER.            (3.01.00)

WINCAM.386      ;READY For Windows 3.1 only!

NCRSDMS.SYS     ;READY For Windows NT.               (3.01.00)

OS2CAM.ADD      ;READY For OS/2.                     (3.00.09)

-------------------------------------------------------------------------------

INSTALLING THE DEVICE DRIVERS

Copying the CAMpliant Device Drivers to the Boot Drive

Although the  host bus adapter's CAMcore firmware can access the hard disk drives attached to the computer independently, the CAMpliant device drivers COSCAM and SCSIDISK act as the interface between your computer system and the CAMcore firmware. Use of t

1. Use the COPY command to copy the device driveers from the /DOS directory on the drivers disk to your boot disk, either a floppy disk or a hard disk (C:). For details about the COPY command, consult your MS-DOS manual. The drivers are:

DOSCAM.SYS (or .386)

SCSIDISK.SYS (or .386)

Modifying the CONFIG.SYS File

1. Add the lines to your system's CONFIG.SYS file in this sequence:

DEVICE=C:<PATH>DOSCAM.SYS (or .386)

DEVICE=C:<PATH>SCSIDISK.SYS (or .386)

The drivers must be listed in this order, otherwise, problems with some applications on some systems may occur.

If the computer system is using DOS's HIMEM.SYS and EMM386 memory managers, the CAMpliant device should be listed after thouse entries. For example, a typical CONFIG.SYS file may look like this with the CAMpliant device drivers:

device=c:<path>himem.sys

DOS=HIGH, UMB

DEVICE=C:<PAHT>EMM386.EXE <OPTIONS>

FILES=30

BUFFERS=15

DEVICE=C:<PATH>DOSCAM.386

DEVICE=C:<PATH>SCSIDISK.386

When installing the DOS device drivers, Ballard Synergy recommends not mixing 386 drivers with SYS drivers. 386 drivers will not work with an 80286 microprocessor. SYS drivers will work with all 80x86 microprocessors but will not take advantage of advance

DOSCAM COMMAND LINE OPTIONS

The DOSCAM device driver has several embedded functions which can be accessed via switches on the command line. These options are avalable with both the SYS and 386 driver versons. These options are described below.

Using the /RAMCOPY Option

RAMCOPY is an embedded command which tells the drivers to load the ROM on the controller card into RAM, thereby increasing the performance of the SCSI ROM (NOTE: although it increases performance, this option will use more RAM). To use RAMCOPY, simply inc

DEVICE=C:<PATH>DOSCAM.SYS /RAMCOPY

or

DEVICE=C:<PAHT>DOSCAM.386 /RAMCOPY

Upon boot, the RAMCOPY command will be executed and ROM will load into RAM. The first ROM will be copied into expanded memory if available.

Using the /T=n Option

DOSCAM uses a timeout mechanism to detect certain errors. When DOSCAM issues a command to a SCSI device, a timer is started. If the timer expires before the command completes, DOSCAM assumes that something has gone wrong with the device, and takes steps t

DEVICE=C:<PATH>DOSCAM.SYS /T=4

or

DEVICE=C:<PAHT>DOSCAM.386 /T=4

with /T being the timeout option and 4 being the number of seconds you want it to wait before timing out.

Using the /EMS Option

This option enables the use of expanded memory if it is available. If software applications experience conflicts in expanded memory, this option should not be used. DOSCAM's default setting is for expanded memory usage to be disabled. DOS's DEVICEHIGH com

DEVICE=C:<PATH>DOSCAM.SYS /EMS

or

DEVICE=C:<PATH>DOSCAM.386 /EMS

Using the /ASK Option

This option prompts the user at system bootup whether to load DOSCAM or not. For example, the line in CONFIG.SYS that loads DOSCAM would look like this:

DEVICE=C:<PAHT>DOSCAM.SYS /ASK

or

DEVICE=C:<PATH>DOSCAM.386 /ASK

Using the /IMAGE=ssss Option

This option is used when the host bus adapter has no CAMcore firmware. The CAMcore image exists at the end of the DOSCAM driver file. /IMAGE will treat the CAMcore image in the file to have been at ROM segment 'ssss'. For multiple host bus adapters, multi

DEVICE=C:<PATH>DOSCAM.SYS /IMAGE=D800

or

DEVICE=C:<PATH>DOSCAM.386 /IMAGE=D800

SCSIDISK COMMAND LINE OPTIONS

The SCSIDISK device driver has several embedded functions which can be accessed via switches on the command line. These options are described below.

Using the /EMS Option

This option enables the use of expanded memory if it is available. If applications experience conflicts in expanded memory, do not use this option. Use of this switch is not necessary if already enabled with DOSCAM. For example, the line in CONFIG.SYS tha

DEVICE=C:<PATH>SCSIDISK.SYS /EMS

or

DEVICE=C:<PATH>SCSIDISK.386 /EMS

Using the /ASK Option

This option prompts the user at system bootup whether to load SCSIDISK or not. For example, the line in CONFIG.SYS that loads SCSIDISK would look like this:

DEVICE=C:<PATH>SCSIDISK.SYS /ASK

or

DEVICE=C><PATH>SCSIDISK.386 /ASK

Using the /UNITS=n Option

SCSIDISK allows the use of removable media, such as cartridge hard drives. The /UNITS=n option sets the number of units reported to MS-DOS to 'n'. This is useful when the system is started without any media in the drive or when media is inserted later (af

DEVICE=C:<PAHT>SCSIDISK.SYS /UNITS=n

or

DEVICE=C:<PATH>SCSIDISK.386 /UNITS=n

Using the /PROTECT=x Option

This option write-protects MS-DOS volumes on a SCSI hard drive (NOTE: this option does not work with an IDE drive). Any write attempt will return a Write Protected Media error code. To enable the write protection option, the line in CONFIG.SYS that loads

DEVICE=C:<PAHT>SCSIDISK.SYS /PROTECT=x

or

DEVICE=C:<PAHT>SCSIDISK.386 /PROTECT=x

Where x is the DOS volume, such as C:, D:, E:, etc. Note: x is  replaced by the volume letter without any colons or commas, for example:

DEVICE=C:<PAHT>SCSIDISK.386 /PROTECT=D

or

DEVICE=C:<PAHT>SCSIDISK.386 /PROTECT=DEF

This protection is not guaranteed as it is possible to bypass the protection with low-level programs, such as FDISK which uses INT 13. Another possible bypass is to boot from a floppy disk without the write protection option enabled.

Trouble Shooting

1. THE COMPUTER LOCKS UP AND WILL NOT COMPLETE BOOTING WHEN READING THE CONFIG.SYS FILE AND THE HARD DISK DRIVES ARE NOT SEEN BY THE DEVICE DRIVERS.

a)      Make sure all the hard disks are configured with different ID numbers.

b)      Make sure the last drive on the SCSI chain is terminated.

c)      Make sure the drivers were installed in the correct sequence.

d)      make sure the drivers' CONFIG.SYS line has the correct path to the drivers.

2. THE DEVICE DRIVERS DO NOT SEE ONE OF THE HARD DISK DRIVES.

a)      Reboot the machine.

b)      Make sure the hard drives have different ID numbers.

c)      Make sure the  controller and the last hard drive are terminated.

d)      Check all cable and power connections.

e)      Make sure the drivers were installed in the correct sequence.

f)      Make sure the drivers' CONFIG.SYS line has the correct path to the drivers.

HARD DISK DRIVE INSTALLATION

Special Information for MS-DOS 3.3 and 4.01 Users:

The Version 3.0 CAMcore on your host bus adapter card supports up to seven hard disk drives without installation of the CAMpliant device drivers. However, due to the limitations of older versions of MS-DOS, only two hard drives will operate reliably when

Installing Hard Disk Drives for Use

This host bus adapter's CAMcore firmware can access the SCSI hard disk drives attached to the computer independently of device drivers. To install a hard disk drive for use, follow these steps:

1. Turn OFF the power to your computer system.

2. Install the hard drive(s) being added to your computer system following the instructions in the device's hardware installation manual.

3. Make sure the device drivers are installed on the bootable drive, either the A: drive (floppy disk) or the C: drive (hard disk), and properly listed in the CONFIG.SYS file. Then switch ON the power to the computer system.

4. After the computer boots, run the FDISK program to partition the hard drive(s) being installed. For details about the FDISK program, consult your MS-DOS manual.

5. When FDISK completes partitioning and your computer has rebooted, run the FORMAT program. For details about the FORMAT program, consult your MS-DOS manual. Be sure to use the /S option to copy the operating system files if you want the disk being forma

6. If you created a new hard disk drive that boots, put the CONFIG.SYS file that calls the device drivers on that drive as well as the device driver programs themselves. Please refer to the section entitled INSTALLING THE DEVICE DRIVERS of this manual.

7. To insure that sufficient drive letters are available to identify all devices connected to the SCSI chain, add the MS-DOS 'LASTDRIVE' command to the CONFIG.SYS file:

LASTDRIVE=x

with x specifying a drive letter in the range C though Z. The letter assigned to LASTDRIVE represents the last valid drive MS-DOS is able to recognize and also represents the maximum number of drives available. For example, LASTDRIVE=K allows access to el

The SCSI hard drive(s) added to your system can now be used.

Trouble Shooting

1. COMPUTER HANGS OR LOCKS UP WHEN LOADING THE CONFIG.SYS FILE AND HARD DISK DRIVES ATTACHED ARE NOT SEEN BY THE COMPUTER SYSTEM.

a)      Make sure that all the hard disks are configured at different ID numbers.

b)      Make sure the last drive on the SCSI chain is terminated.

c)      Check all cable and power connections.

2. MS-DOS WILL NOT BOOT FROM THE HARD DISK

a)      Make sure the primary MS-DOS partition is set active (with FDISK).

b)      Make sure the /S parameter was used when formatting the boot partition.

c)      Make sure the boot drive has the lowest ID number on the SCSI chain.

3. THE DEVICE DRIVERS DO NOT SEE ONE OF THE HARD DRIVES.

a)      Reboot the computer.

b)      Make sure the hard disk drives have different ID numbers.

c)      Make sure the controller and the last hard disk drive are terminated.

d)      Check the cable and power connections.

e)      Make sure the drivers were installed in the correct sequence.

f)      Make sure the drivers' CONFIG.SYS line has the correct path to the drivers.

MS-DOS 5.0

The 3.00 release supports MS-DOS 5.0 similarly to MS-DOS 4.01 with the exception of the following installation feature.

1. When installing the CAMpliant drivers on an 8088 (XT) computer, insert the MS-DOS stacks command, in the CONFIG.SYS file BEFORE the CAMpliant device drivers. For example:

STACKS=8,32

DEVICE=C:<PATH>COSCAM.SYS

DEVICE=C:<PATH>SCSIDISK.SYS

EMM386.EXE

EMM386.EXE, the expanded memory manager included with MS-DOS 5.0 and Windows 3.1, is compatible with the Ballard Synergy CAMcore and CAMpliant software. EMM386.EXE uses extended memory to simulate expanded memory for those applications that use it. EMM386

DEVICE=C:<PATH>EMM386.EXE

must come before the CAMpliant device drivers. The ordering is as follows:

DEVICE=C:<PATH>HIMEM.SYS

DOS=HIGH, UMB

DEVICE=C:<PATH>EMM386.EXE

DEVICE=C:<PATH>DOSCAM.SYS (or .386)

DEVICE=C:<PATH>SCSIDISK.SYS (or .386)

HIMEM.SYS must be included in the CONFIG.SYS file before the device command for EMM386.EXE

Several options are also available for use with EMM386.EXE. Two of the most useful are RAM and EMS. The RAM option provides access to both expanded memory and the upper memory area. The EMS option provides access to the upper memory area but not to expand

Troubleshooting

1. WHILE USING THE XCOPY /V COMMAND TO COPY FILES, YOU GET AN ERROR READING DRIVE C:

a)      Put d=64 after DEVICE=C:<PATH>EMM386.EXE in your CONFIG.SYS file. The d=nnn command specifies how much memory should be reserved for buffered direct memory access (DMA). d=64 indicates that 64K is the largest DMA transfer that will occur while EMM

QEMM

Installing QEMM on a SCSI Drive

As more and more application programs use greater amounts of RAM, the ability to load device drivers into available high memory has become more important. Quarterdeck's Expanded Memory Manager 386 (QEMM) was developed for this purpose and Ballard Synergy

WINDOWS 3.0 & 3.1

Installing Windows 3.0 & 3.1 with SCSI Drivers and Peripherals

1.  Prepare the SCSI hard disk with MS-DOS as described above in the HARD DISK DRIVE INSTALLATION section.

2.  Install Windows 3.0 or 3.1 as per the applicable Windows manual. Windows will make the appropriate alterations to the CONFIG.SYS file to load its drivers.

3.  Reboot the system.

There are no limitations when running Windows 3.0 or 3.1 with the SCSI drivers. Windows 3.0 will run in real, standard, or enhanced modes and Windows 3.1 will run in standard or enhanced modes. See the WINCAM.386 section for further enhancement of Windows

CD-ROM SUPPORT

CD-ROM and SCSI

A CD-ROM drive with a SCSI interface may be connected to the same SCSI controller with other SCSI devices in the system. Follow the manufacturer's instructions included with the CD-ROM drive for hardware installation of the unit.

1.  If it is the only SCSI device in the system, the CD-ROM drive must be terminated. Refer to the CD-ROM drive's manual to perform this function. If the CD-ROM drive is installed in a system with other SCSI devices, and is the last device on the SCSI cha

2.  Included in this package is a CDROM device driver which must be installed in your CONFIG.SYS file to access the CD-ROM drive. The CD-ROM device driver ending in .SYS (CDROM.SYS) can be used with 80286 and higher microprocessors. The CD-ROM device driv

DEVICE=C:<PATH>DOSCAM.SYS (or .386)

DEVICE=C:<PATH>SCSIDISK.SYS (or .386)

DEVICE=C:<PATH>CDROM.SYS (or .386) /D:NAME

The NAME can be any given name for your CD-ROM drive, up to 8 characters long.

3.  To insure that sufficient drive letters are available to identify all devices connected to the SCSI chain, add the MS-DOS 'LASTDRIVE' command to the CONFIG.SYS file. See the HARD DISK DRIVE INSTALLATION section and your MS-DOS manual for further detai

4.  Unless your CD-ROM access software (provided by the disc vendor) specifies, otherwise, you must load Microsoft's CD-ROM Extensions, MSCDEX, from an AUTOEXEC.BAT file in order to access your drive. Add the following line to your AUTOEXEC.BAT file:

MSCDEX /D:NAME

For example, if:

DEVICE=C:<PATH>CDROM.SYS /D:FRED

is in CONFIG.SYS, then:

MSCDEX /D:FRED

must be added to AUTOEXEC.BAT file.

5.  Check installation instructions for the CD-ROM drive itself. There may be other parameters necessary to include with MSCDEX.

6.  When MSCDEX is loaded from the AUTOEXEC.BAT file, a message is returned assigning a drive letter to the CD-ROM drive. For example:

DRIVE D = DRIVER FRED UNIT 0

This informs you that the CD-ROM drive is recognized and ready for use.

7.  The CDROM device driver has embedded functions available, which can be accessed via switches on the command line. These options are as follows:

/EMS Enables the use of Expanded Memory if it is available.

/ASK Prompts the user at initialization time whether to load CDROM or not.

Trouble Shooting

1.  THE CD-ROM DRIVE IS NOT SEEN AT BOOT TIME.

a)  Make sure there is power to the CD-ROM unit.

b)  Make sure there is no ID conflict.

c)  Check the cable and power connections.

d)  Make sure the driver was installed and in the correct sequence with the other drivers.

e)  Make sure the driver's CONFIG.SYS line has the correct path to the driver.

f)  Make sure MSCDEX has the same drive name as the CDROM driver in the CONFIG.SYS file.

WINCAM.386

Windows 3.1 Improved Hard Disk Drive Access

Ballard Synergy's CAM pliant device driver, WINCAM.386, works with the Windows 3.1 operating system to enhance the input/output of the computer's SCSI hard disk drive(s) through improved interaction between the system and the hard disk drive controller. T

Under DOS, when a program accesses a hard disk drive, the following steps occur:

(1)  The application makes an INT 21h call to DOS.

(2)  DOS identifies the location of the file.

(3)  DOS issues an INT 13h call to the hard disk BIOS

(4)  The hard disk BIOS communicates directly with the hard disk controller.

This works well when the DOS ROM BIOS and the SCSI hard disk were both installed and tested by the computer manufacturer. However, SCSI hard disks are often added at a later time and the disk manufacturer must provide a device driver specific to the hard

Under the previous version of Windows (version 3.0), the Windows environment, the 386 enhanced mode, and Windows-compatible programs all run in protected mode but are required to switch to real mode (othrewise known as virtual 8086 mode) in order to execu

(1)  Windows starts in real mode running the application program.

(2)  The application makes an INT 21h call to DOS.

(3)  The 386 enhanced mode traps the interrupt and switches to protected mode.

(4)  Protected mode device drivers look at the call and try to qualify it.

(5)  If the device drivers cannot handle the call, the system switches to real mode to let MS-DOS handle the call.

(6)  DOS identifies the location of the file.

(7)  DOS issues an INT 13h call to the hard disk BIOS.

(8)  The hard disk BIOS communicates directly with the hard disk controller.

On many computers, the resident hard disk drive BIOS is slow and inefficient. To correct this. the Windows 3.1 32-bit disk access takes advantage of advances in disk-access technology that enhance the BIOS. For SCSI systems, the Ballard Synergy CAMpliant

WINCAM.386 Installation

WINCAM.386 is transparent to the user. After WINCAM.386 has been installed and is running, hard disk drive I/O is approximately twice as fast as the system prior to the WINCAM.386 installation. The advantages of using WINCAM.386 can be verified by using t

double.

32-bit access in the Virtual Memory settings of the Windows 3.1 Control Panel do not have to be enabled in order for WINCAM.386 to work.

To use WINCAM.386, the computer system can only have SCSI hard disk drives attached. WINCAM.386 is not compatible with IDE drives. Use of WINCAM.386 with an IDE hard disk drive may result in system crashes and data destruction.

If the system will have more than two SCSI hard disk drives attached, all drives should be partitioned prior to running Windows 3.1 with WINCAM.386. See the MS-DOS User's Manual for information on how to partition hard disk drives with

FDISK.

1.  Copy WINCAM.386 to the WINDOWS directory.

2.  Edit the SYSTEM.INI file. Add the following line in the [386Enh] section:

DEVICE=C:<PATH>WINCAM.386

3.  Ensure that the following lines are also in the [386Enh] section of SYSTEM.INT. If not, add them:

device=*pagefile

device=*pageswap

device=*blockdev

device=*int13

4.  Check the CONFIG.SYS file. Ensure that SCSIDISK.SYS (or SCSIDISK.386) is present. SCSIDISK is necessary for WINCAM.386 to obtain information about the system's SCSI configuration. If it is not present, add the line:

device=C:<PATH>SCSIDISK.SYS

or, if using the 386/486 specific drivers,

device=C:<PATH>SCSIDISK.386

WINCAM.386 is compatible with the DOSCAM and CDROM device drivers.

5.  After editing the CONFIG.SYS file, reboot the computer to start WINCAM.386.

SYTOS PLUS TAPE BACKUP SUPPORT

Installing a SCSI Tape Drive

High-speed tape backup has become essential to computer systems as the capacities of hard disk drives have increased. To provide support for the Sytos Plus Tape Backup System Version 1.2 or newer, Ballard Synergy has developed and provided a device driver

1.  Determine where in the SCSI chain you wish to place the tape drive. If it is the only SCSI device in the system, the tape drive must be terminated. Refer to the tape drive's manual to perform this function. If the tape drive is     installed in a syst

2.  Consult the manuals for the tape drive and set the ID number. Remember that it must have its own individual ID. The tape drive cannot have the same ID as any other SCSI device connected to the SCSI chain.

3.  Use the COPY command to copy the device driver, ASPICAM.SYS, from the /DOS directory on the drivers disk to your boot disk, either a floppy disk or a hard disk (C:). For details about the COPY command, consult your MS-DOS  manual.

4.  Add the ASPICAM.SYS line to your CONFIG.SYS file after ther DOSCAM.SYS and before the SCSIDISK.SYS lines, as follows:

DEVICE=C:<PATH>DOSCAM.SYS (OR .386)

DEVICE=C:<PATH>ASPICAM.SYS

DEVICE=C:<PATH>SCSIDISK.SYS (OR .386)

See above for further information on the differences between the SYS AND 386 versions of DOSCAM and SCSIDISK.

5.  Follow the instructions in the Sytos manual for the installation and operation of the Sytos Plus software.

SUPPOTE FOR ADAPTEC'S

ADVANCED SCSI PROGRAMMING INTERFACE (ASPI)

The Advanced SCSI Programming Interface (ASPI) is a defined protocol by Adaptec for SCSI device drivers and programs to administer I/O requests to host bus adapter-specific ASPI drivers. The ASPI manager oversees host bus adapter resources, providing a ha

Once the ASPI device driver is loaded, the SCSI drivers integrate each type of SCSI device into the operating system through ASPI, independent of the installed hardware. In order to prevent each SCSI driver from having to scan the SCSI bus for devices at

Ballard Synergy has developed several ASPI device drivers to provide the widest possible support for users of CAMcores and CAMpliant device drivers with ASPI-compatible products. For DOS products, ASPICAM.SYS provides a software interface for SCSI periphe